Common NOVELL NetWare Questions & Answers

1.	What does the message 'ASPITRAN loaded re-entrantly' mean?

This message means that ASPITRAN has already been loaded, and does not need to be loaded again.  This is a normal message.  When a module is loaded re-entrantly, it means the data specific to the new instance is retained, but the code is discarded.  The code from the first load will be used.

2.	I am getting the message 'Non-maskable interrupt (parity) error.  	What is the 	problem?

The problem may be due to the RAM SIMM modules used (for example, 3 chip by 1 meg SIMM).  Change it to a 9 by 1 meg or 9 by 4 meg SIMM, depending on the system configuration.  It may also be a bus-master card operating at too high a speed setting, try DMA speed 5.0MB/sec.

3.	Does Adaptec provide a driver for ESDI controllers running 	under NetWare 3.11?

The driver required is ISA.DSK or ESDI.DSK, and is shipped with Novell NetWare.  This driver is not available directly from Adaptec.

4.	I have the AHA-1542B/GIG or the AHA-1742A with the greater 	than a GIG BIOS and my system doesn't see the hard drive or 	just hangs when I run Novell's SERVER program.  What is the 	problem?

The greater than a gig BIOS is a DOS solution and should not be used with NetWare 386, since greater than a gig support is built in the Novell OS. If you have a 1542B/GIG, remove the jumper at J6, position 2.  If you have a 1742A, disable greater than a gig support with the EISA configuration utility.

5.	Why does NetWare report volume sizes larger than the drive's 	capacity?

This is due to extraneous bytes in the FAT area of the disk drive.  This problem is easily resolved by performing a low level format of the drive. Use ADL.EXE for the AHA-1740 series adapters or SCSI_FMT.EXE from the Adaptec BBS.  The format utility is in the BIOS of all the adapters except the AHA-1740. Use DOS DEBUG, and type G=DC00:6.

6.	How do I get the latest NOVELL driver?

Download UPGRAD11.ZIP and NETWARE.ENC from the Adaptec BBS to upgrade your driver to the latest version.

7.	Why are there 7 identical HD's on my screen?

There is a SCSI ID conflict on the SCSI-bus. Make sure that the SCSI ID is 7 for the Adaptec hostadapter and the SCSI ID for the HD is 0.

8.	Under Install.nlm with the AHA-1542C card there is a message 	"The Host Adapter Appears To Be Hung ..."or "There are no 	accessible 	HD ..."

After you reboot your system press Ctrl-A, select Configure/View Host Adapter Settings, then press function key F6 to reload the defaults. (The >1G option was enabled and this will not work under NOVELL)

9.	I have more then 16M of Ram with a AHA154xB or C, does 	Adaptec support this under Novell 3.11?

Yes, you need a driver version 2.24! To get the driver call our BBS and download Upgrade.exe. (You still need to use the load command above16=y)