Greetings from MEL's Diner BBS!       (714) 686-1522

IP is a serial interrupt and port verification program. Just run "ip" and the

program will check the RAM bios data area to see what port addresses your

system is currently aware of. The program will then run a diagnostic routine

to discover any additional ports, and update the data area accordingly. In the

meantime, IP will also tell you which IRQ's are being used by what ports.

NOTE: IP does not assume that address 0x3F8 is connected to IRQ3, 0x2F8 to

IRQ4, etc... as several other serial port diagnostic utilities do. IP acually

enables the 8250's interrupts and does a functional check of the 8259 interrupt

controller chip and it's associated circuitry. In doing so, IP determines

exactly which hardware ports are connected to which IRQ's.

IP will now take a command line switch (/n or /x). The /n (default) switch tells

IP to check standard IBM port addresses (3F8h,2F8h,3E8h,2E8h). The /x switch

will allow IP to look for additional addresses which are sometimes used in non-

standard hardware schemes. The /x switch should be used with caution as some

non-standard hardware layouts will crash unexpectedly when one of these add-

itional port addresses are searched by IP.

EXAMPLE:

Interrupt and Port Verifier (serial) - Version x.xx

Copyright (C) 1990-1991, AM Research Co.

Bios reports --> COM1:3F8 COM2:2F8  2 serial ports

PART#  BASE RD IE ID LC MC LS MS SP LSB MSB  IRQ (XT) IRQ (AT) INT  VECTOR

01234567 89ABCDEF

8250-B 3F8: 00 00 01 03 00 62 08 FF 030 000  00001000 00000000 0Ch 119C:032A

8250A  2F8: 0D 00 01 03 00 60 F0 00 030 000  00010000 00000000 0Bh 119C:02A2

16550A 3E8: 00 00 01 03 00 62 30 00 006 000  00000100 00000000 0Dh 119C:02FA

3 serial ports found!  Updating bios data area.

EXPLANATION:

The first info line after the copyright notice tells us that the system

bios data area (RAM) thinks that there are two serial port addresses in-

stalled in the computer under test. These addresses are found by the bios

POST routines when the system is first booted. Many older bios test routines

only check for COM1 and COM2 as the example shows.

After reading the bios, the program prints a list of each port address that

it found to contain a serial port. Each line will begin with the part number

of the UART chip which was found. After the part number the port address is

shown followed by the contents of the port's ten readable registers. The

values displayed may make more sense if you check the 8250 data sheet for

register descriptions, etc.

Following the register values is a list of 1's and 0's indicating which

hardware IRQ the port in question is connected to. This is the place to look

if you are having a problem that you suspect is due to an IRQ conflict.

Generally speaking, it is ok for two ports to share the same IRQ as long as

your work only uses one of the hardware devices at at time.

After the IRQ block is the interrupt number and vector of the ISR that will

be called to service the port if an interrupt is generated. Note that almost

all serial ports will have their interrupts turned OFF at the time you are

performing the test. The interrupt vector that is displayed will probably be

changed by an application that enables the port's interrupts.

It is very important for the operator to realize that IP is a diagnostic

utility which is meant to be used in checking/troubleshooting a newly modifed

or questionable computer. As such, IP may therefore interfere with the system's

operation as it actively enables and operates each port's interrupt circuitry.

IP will probably NOT work properly if you have loaded any type of software that

activates the port's interrupts such as fossil drivers, etc. IP may very well

cause a crash if used with any type of multi-tasking software.

Finally, IP will modify the bios data area to report any additional ports it

discovers.

I would be quite interested in hearing from anyone with comments, suggestions,

crashing reports, etc...

As you might imagine, IP remains Copyright (C) 1990-1991, AM Research Co.

Hope you enjoy using it.

bye for now...

MEL

HISTORY

Version 1.22 - Fixed bug in the bios data area update routine. (It didn't

always update it!)

Version 1.21 - Fixed bug whereby NS16550's and NS16550A's wouldn't produce

an interrupt if the fifo buffers were activated. Also added

NS16552 detection.

Version 1.20 - Added extended base addresses. Also protected system from the

isr test if interrupts are already activated.

Version 1.10 - Rewrote the isr. Works much better now.

Version 1.04 - Yet more bugs.

Version 1.03 - More minor bugs.

Version 1.02 - Minor bug fix.

Version 1.01 - First release.