To generate the bdos modules -

{c}a10 rwa		; assemble the assembler module

{c}bdosc @allc		; compile all the C modules

NOTE -- has the code used to clear the bss for program load been speeded up?

History of Changes made to GEMDOS V1.1.

---------------------------------------

05 Jun 87  ACH	PROC.C		Problem - After physical disk error COMMAND.PRG

gives "Command not found." message for all

subsequent external commands.

Fix - Changed the error handling code in

xexec() to always do an ixterm(t); on any disk

error, thus freeing memory.

26 Jun 87  ACH	TIME.C & BIOS	To enable GEMDOS to take advantage of real-time

clock/calendars a new BIOS function (11h)

has been added. Whenever the BDOS sets a new

time or date this function is called and, if

the hardware supports it, the real-time clock

can be updated. When the BDOS needs the current

time or date the function is called to allow

the BIOS to update the internal BDOS values.

If there is no hardware clock the BIOS can

simply ignore this call as the BDOS still

maintains the time and date internally based

on the system tick.

16 Sep 87  ACH	SUP.C		Problem - the internal date and time variables

used to time stamp file updates are set to 0

when the OS starts up.

Fix - Changed cinit() in SUP.C to call new BIOS

date_time() function (11h) so that the BIOS can

initialise the internal variables from a

hardware real-time clock, if supported.

9 Oct 87 ACH	BDOSMAIN.C	Duplicate definition of supstk and SUPSIZ

removed from this module. (Duplicated in

PROC.C and FS.H.)

If the supervisor stack is to be OEM

configurable some more reliable method must be

devised.

19 Nov 87 ACH	RWA.A		Added _lbmove, _bfill, _bmove routines.

(These were supposed to be in something called

TOOLS.ARC which was not supplied by DRI.)