This is a directory containing definitions of about 40 units.  The units are:

brlycrn (barleycorn)                 8.46668360002E-3_m

buUK (UK bushel)                           .036368736_m^3

cable                                   219.456438913_m

circle                                  6.28318530718_r

cwt (hundred-weight)                        45.359237_kg

dozen                                              12

dram                                 1.77184519531E-3_kg

drfl (fluid dram)                    3.69669119531E-6_m^3

drt (troy dram)                           .0038879346_kg

drUK (UK dram)                         3.551634375E-6_m^3

frlng (furlong)                         201.168402337_m

gal-d (dry gallon)                       .00440488375_m^3

gi (gill)                             1.1829411825E-4_m^3

gr-gross (great gross)                           1728

gross                                             144

hand                                      .1016002032_m

hhd (hogshead)                          .317974589856_m^3

inUS (US inch)                       2.54000508001E-2_m

league                                           5556_m

link                                    .201168402337_m

minim                                6.16115199218E-8_m^3

minimUK (UK minim)                   5.91939062502E-8_m^3

pkUK (UK peck)                             .009092184_m^3

pwt (pennyweight)                        .00155517384_kg

pt-d (dry pint)                       5.5061046875E-4_m^3

ptUK (UK pint)                            .0005682615_m^3

qt-d (dry quart)                      1.1012209375E-3_m^3

qtUK (UK quart)                            .001136523_m^3

quadrant                                1.57079632679_r

quarter                                   11.33980825_kg

RPM (revolutions per minute)             .10471975512_r/s

score                                              20

scrUK (UK scruple)                     1.183878125E-6_m^3

scruple                                   .0012959782_kg

sextant                                  1.0471975512_r

sgn (sign)                              .523598775598_r

stone                                      6.35029318_kg

ydUS (US yard)                          .914401828804_m

All of these units are organized in a custom directory.  For convenience, the

custom directory also repeats some of the built-in units.  It should be noted

that some of these units have multiple definitions (for example, the cable is

variously defined as 120 fathoms, 100 fathoms or 608 feet).

One unit, RPM, is a replacement for the built in rpm unit.  The new RPM

contains an angle unit in the definition and, will work properly in

conversions to, or from, radians per second and degrees per second.

=============================================================================

Eric Haas    Internet: ERICHAAS@delphi.com    FidoNet: Eric.Haas@1:261/1000

EHaas@aol.com

=============================================================================