Product Name    : EPATITST.EXE v1.01

Description		: EPAT Functional Test Software

Date of release : 27th September, 1996

Hardware requirements:

(1) IBM/Compatible PC

(2) SMC666/665 based Parallel Port Adapter

(3) Shuttle EUT (EPAT/EPEZ/EPAT+  Adapters)

(4) Shuttle passive/active pass-through loop-plug

(see Pasplug.prn file)

(5) An IDE Hard-Disk drive as master

(6) required power and data cables.

Software requirements:

DOS Operating System (ver 5.x and above)

The functional test software performs various tests on the EUT

adapter and validates its different functionalities.

These tests include the following:

IRQ DETECTION TEST :

This test will initialize and check the EUT for the presence

of  default IRQ values. If the check fails  then this test will report

an error condition.

READ TESTS :

These test the data path between the adapter and the parallel port in

various transfer modes such as Nibble, Uni-Directional, Bi-Directional,

and EPP.

WRITE TESTS :

These test the data path between the adapter and the parallel port in

various transfer modes such as Nibble, Uni-Directional, Bi-Directional,

and EPP.

EPPROM ACCESS TEST :

This will validate the read/write operations with the EEPROM

on the EPATI adapter. Upon successfull completion, this test also

initializes the EEPROM to the character string found in the

configuration file, EEPROM.CFG.

This file is required to be present in the current working

directory. Otherwise this test will be skipped.

PASS THROUGH TEST :

This test requires that Shuttle passive/active  pass-thru loop-plug

be connected to the pass-through port of the EUT. Various tests

to confirm the proper connection of data, status and control signal are

carried out.

During this test the user will be asked to press 'y'es or 'n'o

depending on the condition of LEDs in the passive loop plug.

If an active loop-plug is connected the above user interface is

not required.

NEGOTIATION TEST :

This test will check the EUT for the EPP mode

initialization in Daisy disabled condition.

TEST RESULTS :

The test also keeps an account of the number of adapters tested,

passed, and failed.

COMMAND LINE PARAMETER :

None .

TIME TAKEN FOR TESTING ONE UNIT

Minimum of 32 seconds with passive loop-plug.

Minimum of 25 seconds with active  loop-plug.