ZPC Upgrade Disk Instructions

Table of Contents

Upgrading ZPC ............................ 2

Two drive upgrade ................... 2

Single drive upgrade ................ 2

Winchester upgrade .................. 3

ZPC Support Programs ..................... 3

ZHSTEST ............................. 3

VIDCON .............................. 3

INT14 ............................... 4

M2TO3 ............................... 6

BLINK ............................... 6

SETZPC .............................. 7

The ZPC Interrupt Patches ........... 7

Application Program Support

PATCHER ............................. 7

Notes on Application Patches ........ 7

Special Application Support ......... 8

FIXLTS and FIXSYM ................... 8

FIXIFF .............................. 9

FIXWI ............................... 9

Using Framework Version 1.1 ........ 10

Using Javelin Version 1.1 .......... 10

1

ZPC Upgrade Disk Instructions

This  disk  contains  patches to upgrade  ZPC,  support  programs

for  ZPC,  and patches for certain application programs to  allow

them to be run under ZPC.

Upgrading Your ZPC

The  ZPCUP.COM program on this disk is used to patch your ZPC.COM

file from version 2.0.3 to version 2.1.3.  Similarly, ZPCSMUP.COM

is  used to patch ZPCSM.COM from version 2.0.2 to version  2.1.2.

IMPORTANT:   You  should retain a copy of the original version  2

software.  It is not likely,  but you may find that some programs

will  run better under version 2.0 than under 2.1,  so you should

have version 2.0 available just in case.

Upgrading Your ZPC -- Two Drive System

To upgrade your copy of ZPC in a two drive system,  follow  these

steps:

1.   Boot up with MS-DOS.

2.   Remove  the  MS-DOS disk from drive A:  and replace it  with

this disk.

3.   Place your ZPC system disk (containing ZPC.COM or ZPCSM.COM)

in drive B:.

4.   Enter

ZPCUP B:       (or ZPCSMUP B: if you are using ZPCSM.COM)

and hit RETURN.

5.   Re-boot, using your ZPC system disk.

Note:   If  your  ZPC file has another name  besides  ZPC.COM  or

ZPCSM.COM,  you  can  specify  the name in the ZPCUP  or  ZPCSMUP

command line, as in

ZPCUP B:ZPCX.COM

Upgrading Your ZPC -- Single Drive System

To  upgrade  your copy of ZPC in a single  drive  system,  follow

these steps:

1.   Boot up with MS-DOS.

2.   Remove  the  MS-DOS disk from drive A:  and replace it  with

this disk.

3.   Copy  ZPC.COM or ZPCSM.COM from your ZPC system disk to this

disk.   You  will have to do some disk swapping,  but should

wind up with this disk in your drive.

4.   Enter

ZPCUP          (or ZPCSMUP if you are using ZPCSM.COM)

and hit RETURN.

5.   Place your ZPC system disk in drive A:, and copy the patched

2

ZPC.COM or ZPCSM.COM from this disk to it.

6.   Re-boot, using your ZPC system disk.

Upgrading Your ZPC -- Hard Disk (Winchester) System

To upgrade your copy of ZPC on a Winchester disk  system,  follow

these steps.

1.   Boot  up and log onto your ZPC partition  and/or  directory,

containing ZPC.COM or ZPCSM.COM.

2.   Place this disk in drive A:.

4.   Enter

A:ZPCUP        (or A:ZPCSMUP if you are using ZPCSM.COM)

and hit RETURN.

5.   Re-boot your system and load in your new ZPC.

NOTE:   The changes made by ZPCUP or ZPCSMUP are  permanent,  and

the  upgrade process does not have to be repeated unless you lose

your  upgraded copy of ZPC.COM or ZPCSM.COM.

ZPC Support Programs

This disk contains programs to support ZPC and the latest version

of  the  ZPC  Hardware Support  circuit  (ZHS  board),  including

commercial versions such as the "Scottie Board".   A schematic of

the do-it-yourself version of the ZHS board is included with this

package.   If you are thinking of building the board,  you should

use  the  included schematic instead of the schematic in the  ZPC

manual.   Note:  The new schematic is for H/Z-100 computers only,

not ET-100 computers.

ZHSTEST -- The ZHS Circuit Test Program

If you have a ZHS board, either home-built or commercial, you can

use  the  ZHSTEST  program on this disk to test  it.   Just  copy

ZHSTEST.COM from this disk to your ZPC system disk or  partition,

and enter

ZHSTEST

and hit RETURN to run it.  It will report on the condition of the

three  ports  that are emulated by the ZHS  circuit.   The  video

control  port  is the section of the circuit that drives the  VI6

line  on the S-100 bus.   The keyboard port is the  section  that

drives the VI7 line.   The video retrace port is the section that

drives the DI0 and DI3 lines.

VIDCON -- The Video Control Port Controller

The combination of the latest ZHS design and ZPC version 2.1  may

not  work  properly with some PC programs.   This is because  the

video control port sometimes does not react fast enough.   If you

have  a program that works under ZPC version 2.0,  but not  under

3

version  2.1,  you can use the program VIDCON.COM to disable  the

video control port,  and try the program again under version 2.1.

To use VIDCON.COM,  copy it to your ZPC system disk or partition,

and enter

VIDCON OFF

at  the  system prompt,  and hit RETURN.   This turns  the  video

control port off, and you can now try your program that would not

run  properly.   To restore operation of the video control  port,

enter

VIDCON

at  the  system promt,  and hit RETURN.   Note:   A symptom of  a

program  that does not work because of the video control port  is

that  the  screen will switch from the 80 column mode to  the  40

column mode when it is not supposed to.

INT14 -- The Serial Port Driver

If you have a commercial ZHS board containing one or more  serial

ports (COM ports), the INT14 program on this disk will provide an

environment  that will allow software to use those ports.   INT14

performs  two  tasks:   A)  It processes  the  BIOS  serial  port

interrupt,  interrupt  14H,  using the board's ports rather  than

mapping serial I/O to the H/Z-100's ports,  as ZPC normally does.

B)  It maps hardware interrupts generated by the serial ports and

passed  into the system via S-100 interrupt lines  to  interrupts

that the PC software is expecting to "see".

In order to the make best use of INT14,  it is important that you

understand  how ZPC maps I/O requests.   It maps them on two lev-

els,  with INT14 providing a third level of mapping, as explained

below:

Level  1 mapping -- MS-DOS I/O requests.   On a PC  computer,  an

application  program can access one of two possible serial  ports

using DOS file I/O commands,  with COM1 or COM2 as the file name.

Similarly,  the program can access parallel ports using LPT1 LPT2

as the file name.   Neither the COMx nor the LPTx file names  are

supported by the H/Z-100 version of MS-DOS,  so ZPC maps the file

name COM1 to PRN, and the file name COM2 to AUX, and it also maps

LPT1 to PRN and LPT2 to AUX.  The CONFIGUR program can assign PRN

and  AUX  to any of the ports on the back of your  computer,  but

normally  PRN  is assigned to either the J1 serial  port  or  the

parallel port, and AUX is assigned to the J2 serial port.  IMPOR-

TANT  NOTE:   Level 1 mapping is always performed by ZPC while it

is in the PC mode regardless of whether INT14 is active or not.

Level 2 mapping -- BIOS I/O requests.   Most application programs

that  are  designed  for use on  a  PC-type  computer,  with  the

important  exception of modem programs and mouse drivers,  access

I/O devices at the BIOS level.  The serial ports are accessed via

a  software interrupt,  interrupt 14H.   The number of  the  port

desired is passed in the DX register, with 0 indicating COM1, and

1 indicating COM2.  Normally, ZPC maps these requests the same as

it  does with level 1 accesses.   A COM1 request is mapped to the

4

PRN device, and a COM2 request is mapped to the AUX device.  When

INT14 is installed, it can map either COM1 alone or both COM1 and

COM2 to actual serial ports on a ZHS board.   The mapping done by

INT14 can be turn on or off after it has been installed.   It  is

important  to  note  that the presence of INT14 does  not  affect

ZPC's  mapping of I/O via the LPTx devices,  so a  program  could

simultaneously use a mouse via a ZHS board serial port, and drive

a  serial printer on one of your H/Z-100 serial  ports.   At  the

BIOS  level,  INT14  supports  hardware handshaking in  a  manner

similar to an H/Z-100 PC computer.  This will be explained later.

Level  3 mapping -- direct hardware access.   Modem programs  and

mouse drivers usually access the serial port hardware directly on

a PC-type computer,  and that is why they will not work under ZPC

unless you provide the PC-style hardware on a board.   The built-

in  serial ports on the H/Z-100 are not compatible with PC serial

ports.   Some programs command the serial port hardware to  gene-

rate  interrupts when a character is received and/or transmitted.

ZHS  board serial ports can be jumpered to pass these  interrupts

on to the system via S-100 interrupt lines, but the actual inter-

rupts generated by these lines are not the same as the interrupts

generated  by  PC serial ports.   INT14 maps  the  interrupts  by

generating  a  software interrupt of the proper type each time  a

hardware interrupt is generated by the ZHS board serial port.

Using INT14

To  use INT14,  copy INT14.COM from this disk to your ZPC  system

disk and/or partition.   After ZPC is loaded,  you can load INT14

by typing

INT14 n

at the system prompt, and hitting RETURN, where n is the number 1

or  2.   NOTE:   YOU  SHOULD INSTALL INT14 WITH ZPC IN THE  Z-100

MODE.   If you install it with ZPC in the PC mode, an area in the

BIOS  RAM segment will not be properly updated unless you  switch

to the Z-100 mode and back to the PC mode.

If you enter INT14 1, BIOS I/O requests to COM1 will be mapped to

the ZHS board port,  but requests to COM2 will still be mapped to

AUX.   If you enter INT14 2, both COM ports will be mapped to the

board.   After INT14 has been installed,  you can turn it off  by

entering

INT14 OFF      (or INT14 O)

When you do this,  mapping of COM1 and COM2 is done by  ZPC.   To

restore mapping by INT14, enter

INT14 Hn

where  n  is a number (0-3) that describes the type  of  hardware

handshaking  desired.   You can also enter INT14 Hn when INT14 is

already  turned on.   The numbers assign hardware handshaking  as

follows:

5

Number entered      Handshaking type supported

0                   No handshaking.

1                   Handshaking via the CTS line.

2                   Handshaking via the DSR line.

3                   Handshaking via the CTS and DSR lines.   This

is  the  default mode active  when  INT14  is

first started, and is the normal IBM PC mode.

If a program attempts to transmit a character to a serial  device

using  interrupt  14H while a handshaking method is  active,  the

character  will  not be transmitted until  the  selected  line(s)

become active (high).  If the line(s) do not become active within

a  specified  time  out  period,  INT14 returns  control  to  the

application program with an error flag set.   The time out is set

to 10 seconds, but can be altered by modifying the INT14 assembly

source code,  and re-assembling INT14.   The hardware handshaking

type you select only affects use of the serial port via interrupt

14H, and has no affect on direct access.

M2TO3 -- A Program to Force Use of Color Text

Occasionally  you may come across a PC program that was  designed

to run in a text mode in color,  but it runs in monochrome (green

or  yellow for normal intensity,  white for high intensity) under

ZPC.   The problem could be that the program is operating in mode

0  or 2 (40 or 80 column black and white text) instead of mode  1

or  3 (40 or 80 column color text).   The M2TO3 program can  take

care of the problem by causing a request to set mode 0 to  result

in  mode 1 being set,  and by causing a request to set mode 2  to

result  in mode 3 being set.   To use M2TO3,  copy M2TO3.COM from

this disk to your ZPC system disk, and enter

M2TO3

to  load the program.   It will remain active until you  re-boot.

NOTE:  Once you load M2TO3, you cannot use SETZPC to make changes

to  ZPC.   You  should run SETZPC and make any  changes  required

before you load M2TO3.   Also,  if ANSISYS is to be used, it must

be loaded before M2TO3.

BLINK -- A Program That Controls Cursor Blinking

If  you prefer a non-blinking cursor,  you can have one while ZPC

is in the PC mode by using the BLINK program.   To use  it,  copy

BLINK.COM to your ZPC system disk, and enter

BLINK OFF      (or BLINK O)

to turn blinking off, or enter

BLINK

to  restore blinking.   NOTE:   This version of BLINK is for  use

only with ZPC.COM (not ZPCSM.COM) after it has been upgraded with

ZPCUP.COM.   Use  with any other version of ZPC could cause  your

system to crash, requiring a re-boot.

6

SETZPC -- The ZPC Control Program

Early  copies of ZPC Version 2 contained a version of the  SETZPC

program  that had a "bug" in it.   If you changed the  monochrome

normal  intensity color from green to yellow and then wrote  your

ZPC  changes to disk,  the color change was not  recorded.   That

problem  is  corrected in the SETZPC program on this  disk.   You

should replace your SETZPC with the one on this disk if you  have

experienced the color problem.

The ZPC Interrupt Patches

ZPC enables some interrupt lines on the S-100 bus in your H/Z-100

computer  that may have never been enabled before.   If you  have

tried  to use ZPC on your computer and simply could not get it to

run at all, it may be that you have a hardware problem in your S-

100  interrupt circuitry.   To help you determine if that is  the

problem,  patches  have  been  added  to  PATCHER.DAT  that  will

completely  remove all code from ZPC that has anything to do with

S-100  interrupts.   The  patches are for ZPC.COM  and  ZPCSM.COM

after  they have been upgraded to version 2.1.   Item 57  in  the

current PATCHER menu is the patch for ZPC.COM, and item 58 is the

patch  for  ZPCSM.COM.   If you make these patches and ZPC  works

where it would not work before, you may have a hardware problem.

Application Program Support

This  disk  contains  support to help you run  more  applications

under ZPC than you could previously.   The support consists of  a

new  PATCHER with more patches,  and some specialized  utilities.

If you choose not to build or buy a ZPC Hardware Support circuit,

you  will need to patch many application programs in order to get

them to run under ZPC.   A few applications will require patching

even if you have a ZHS board.

PATCHER -- An Application Program Patcher

The  patcher program provided on this disk is improved  over  the

original  one  in that it pauses after a screen full  of  patches

have  been  displayed  on the menu.   You can  hit  RETURN  after

reading the screen to see more patches.  When you are prompted to

enter the number of the program to patch, you can enter RETURN to

re-display  the  patches  from the beginning of  the  list.   The

PATCHER.DAT  file on this disk contains a number of  patches  not

included  in  the original,  and some improved versions of  other

patches.  You should copy the new PATCHER.COM  and PATCHER.DAT to

your ZPC system disk, to replace the old ones.

Some Notes on the Application Patches

The patch for Lotus release 1A in PATCHER.DAT is a new  one,  and

includes  a  patch to cause the cursor to be in the  right  place

during editing.  For those of you using the latest version of the

ZHS circuit, a Lotus patch is included that makes only the cursor

patch.   If you have been using the old patches,  perform the new

7

patches on a copy of your Utility disk,  and then re-install your

Lotus drivers.

QuickBASIC  runs  under  ZPC version 2.1 using either  the  patch

provided  in  the new PATCHER.DAT,  or a ZHS  board.   Some  menu

options  in QuickBASIC are selected using the space  key.   Under

ZPC,  you  may  have  to press the space bar twice  to  make  the

selection.

Some  products,  such as Word Perfect and  Multimate,  have  been

released several times under the same version number.  There will

be more than one patch for products like this in PATCHER.DAT.  If

you try a patch and it does not work,  you should start over with

the original unpatched software before trying another patch.

IMPORTANT:   If you patch a program, and it does not run properly

after  the  patch,  you  should always start over  with  a  fresh

unpatched copy of the program before you try other patches,  etc.

Never  re-patch a patched program,  and if you have someone  else

try  to  patch a program for you,  always provide him/her with  a

fresh unpatched copy.

Special Application Support Programs

The special application programs on this disk provide support for

Lotus  1-2-3  release  2 and Lotus Symphony.   There  is  also  a

general  purpose utility that allows the use of programs that use

interrupt 0FFH (255),  and a program that alters the column width

variable.

FIXLTS and FIXSYM -- The Lotus Patch Programs

FIXLTS and FIXSYM can be used to patch Lotus 1-2-3 release 2  and

Symphony  so  that they will run under ZPC without a  ZHS  board.

Even  if you have a board,  you may want to install the  patches,

because  the cursor will not be in the right place during editing

without the patches.   To make the patches, run the Lotus Install

utility  starting  with  a fresh uninstalled  copy  of  1-2-3  or

Symphony.   You can put ZPC in video mode 7 (PC 7) to allow 1-2-3

or Symphony to run while you are running Install.  Answer prompts

and questions as follows:

First Time Installation

Can your monitor display graphics? Yes

How many monitors do you have? One Monitor

IBM color card, single color monitor (see note)

(Answer printer questions as required)

Do you want to name your driver set? No

Note:   With Symphony, you can also select "IBM color card, color

monitor" from the monitor list.   After you have finished running

Install, exit from 1-2-3 or Symphony, place this disk in drive A:

and your 1-2-3 or Symphony System disk in drive B:, and enter

FIXLTS B:      (or FIXSYM B:)

and  hit  RETURN.   If  you have only one  drive,  you  can  copy

8

FIXLTS.COM or FIXSYM.COM from this disk to your Lotus  disk,  log

on to the disk, and enter

FIXLTS         (or FIXSYM)

You should repeat the patch procedure for other disks in your set

on  which  you installed drivers while running Install.   If  you

answer  Yes to the last Install question and give your  driver  a

name  of  your  own,  you can patch it with FIXLTS or  FIXSYM  by

specifying the name in the command line, as in

FIXLTS B:DRIVER.SET

If  you  run  Install again to change  something  (such  as  your

printer  configuration)  after using FIXLTS or  FIXSYM,  it  will

create a whole new driver file, which must be patched again using

FIXLTS or FIXSYM.

FIXIFF -- Allows Use of Interrupt 0FFH

A  few  programs  use  interrupt 0FFH  as  an  internal  software

interrupt.  The vector for this interrupt is used as a pointer on

an H/Z-100, so the interrupt normally cannot be used.  The FIXIFF

program  will allow the interrupt to be used,  while  maintaining

the pointer for your H/Z-100.   To use FIXIFF, copy FIXIFF.COM to

your ZPC system disk, log on to the disk, and enter

FIXIFF

and hit RETURN.  FIXIFF will remain active until you re-boot.

FIXWI -- Fixes the Column Width Variable

When  ZPC is in the PC mode,  it maintains a variable  in  memory

that  contains  the number of characters per line on  the  screen

(either  40  or  80).   This variable is maintained at  the  same

location  in memory as on a real PC,  but the byte following  the

width  variable  is  different,  because it is  part  of  a  jump

instruction used by PSC (Print Screen) utilities.   On a real PC,

the  byte  following  the width variable is  zero,  which  allows

program to access the variable as a word as well as a byte.  If a

program  attempts  to access the width variable as a  word  under

ZPC,  it will not work properly.   The program FIXWI.COM on  this

disk  can be used to temporarily fix the width variable.   To use

it, first put ZPC in the PC mode, and enter

FIXWI

at the system prompt,  and hit RETURN.   This will clear the byte

following the width variable to zero, and it will remain that way

until  you  set the Z-100 mode and then return to  the  PC  mode.

Note:   You must not try to use a PSC utility to print the screen

while  FIXWI  is active,  but you can print text screens with  no

PSC utility loaded while in the PC mode.

9

The symptom of a program trying to access the width variable as a

word  is  that a single character will be printed over  and  over

until the screen is filled, and the program may hang up.

Using Framework II Version 1.1

Use PATCHER as directed in your ZPC manual to apply the FRAMEWORK

II v.  1.1 patch to System Disk no. 2 of the Framework set.  Then

put  ZPC  in mode 7 (type PC 7) and run the Framework SETUP  pro-

gram.   Select item 1 from the first menu if you have not already

installed Framework for use on floppies or your hard disk.   When

you get to the list of screen drivers,  select item 4 (All  other

IBM drivers).   On the list of IBM drivers,  you can select 2, 3,

or 4.   If you select item 2,  you will have to put ZPC in mode 7

each time you want to run Framework.   If you select 3 or 4,  you

will be running Framework in the default PC mode (mode 3).

When you get to the printer port menu, select number 1 (parallel)

regardless  of  whether your printer is actually connected  by  a

parallel  or  serial cable.   ZPC will take care of  mapping  the

Framework printer driver to your printer.

When you have completed the setup procedure, run SETUP again, but

select  item  2 from the first menu.   When you get to  the  Main

menu,  select  item 4 (Telecommunications),  and from that  menu,

select item 4 (Port Addresses).   If you do not have any  PC-type

serial  ports  installed in your computer (such as  the  optional

ports on the Scottie Board),  set all of the port base and vector

addresses to 0.  If you have one PC serial port, set the base and

vector addresses for ports 2 and 3 to 0.   If you have two ports,

set  only the port 3 addresses to 0.   Now type M to get back  to

the main menu, and select item 7 to save your changes.

After you have run SETUP, you will have to patch the file it made

using the FIXFWII program that comes with ZPC Version 2.   To use

FIXFWII,  copy it to your Framework system disk 2,  log on to the

disk, and enter

FIXFWII

and hit RETURN.  Framework II will now be ready for use with ZPC.

Using Javelin Version 1.1

Javelin  will  not  run  in the default PC mode  (PC  3)  without

patching  under  ZPC,  even if you have a  ZHS  board  installed,

because  it accesses the video retrace port on the board using  a

word  input instruction (IN AX,DX) instead of a byte instruction.

The  ZHS  board normally toggles a couple of bits each  time  the

retrace port is accessed so that the software sees the bit in the

opposite state each time it looks at the port.  However, when the

IN AX,DX instruction is used, the bits are toggled twice for each

access,  and the software always sees the bit at the same  state,

and hangs up.

To  prepare Javelin for use under ZPC,  Set ZPC in the default PC

mode  (PC 3),  and run INSTALL (if you want to put Javelin  on  a

10

hard disk or an 8" disk (treat it like a hard disk), and then run

JCONFIG.   When  JCONFIG displays three squares on the screen and

asks if you see red,  blue,  and green squares,  you can answer Y

even  if you have a monochrome screen,  but color memory in  your

computer.  That way, you will see gray scale.  Next, JCONFIG will

display light and dark yellow squares and ask if you see them  as

different  colors.   It does not matter how you answer that ques-

tion.  When it asks you to choose the manufacturer of your screen

adapter,  select the first item (IBM).   When it asks the kind of

IBM adapter you have,  also select the first item (Color Graphics

Adapter).  You can answer the printer questions as required.

After  you  have  run JCONFIG,  make the  required  patches  with

PATCHER to your startup and program disks if you are running from

floppies.  If  you  are  running from a hard disk  or  8"  drive,

temporarily   copy  PATCHER.COM  and  PATCHER.DAT  to  your   JAV

directory,  log on to the JAV directory,  make both patches,  and

then erase PATCHER.DAT and PATCHER.COM from the JAV directory.

If  you  want to run Javelin in video mode 7 (PC 7),  you do  not

need to install the above patch.   You should run JCONFIG in mode

7, and when it asks what kind of IBM adapter you have, select the

last item.

11