Document # HW0001

Adaptec Host Adapter Hardware Commonly Asked Questions  3/93

Q.  Why does my computer/SCSI HA think that I have 7 hard drives,

(or CD-ROMs, or ..) when I only have one drive connected?

A.  When installing an Adaptec SCSI HA, SCSI bus protocol must

be observed.  The HA should be SCSI ID 7, always the highest ID

number, so that the HA will always win SCSI bus arbitration.

SCSI bus theory dictates that each device (the HA is a device)

must have a unique and separate SCSI ID number.  If a HD or

other device is attached to the SCSI bus with the same ID as the HA,

then the HA will see a response for "phantom" devices at IDs where no

device exists.  To solve this problem, simply set the devices ID to

something other than that of the HAs SCSI ID.  If the device is a

boot drive, then the SCSI ID should be set at 0.

The 152x/154xB/1640/174x (in standard mode) BIOS controls hard drives

at SCSI ID 0 and 1 only.

The 174x BIOS in enhanced mode will control hard drives at SCSI ID's 0

through 6.  Hard drives or SCSI devices using ID's 2 through 6

need device drivers to be controlled using the 152x/154xB/1640 or

174x in standard mode.

Q.  Why can't I use the entire size of my hard drive that is

rated over 1 Gigabyte in capacity, when I am using DOS?

A.  DOS 4.x and above has a limitation of 1024 cylinders per

physical drive.  This limitation, on current disk controllers

and SCSI HAs that utilize the standard of 1 Megabyte per

cylinder, leads to a 1 Gigabyte limit.  Only our 1542B/GIG and

174xA (with special BIOS in Enhanced Mode using ASW-C174 v3.1)

have the ability to break this limit.

If your 154xB has a BIOS part number of 420418-00D

or greater, or if your 174xA has a BIOS part number of

450216-00-A or greater, then you already have support for more

than 1 Gigabyte.  Refer to your User's Manual addendum for help

on setting up this feature.  Alternatively users

who need this support can download the latest 154xB or 174x

microcode/BIOS files from this BBS for use with an EPROM burner.

The EPROMs needed are 27C128 at 200ns for the 154xB and 150ns

for the 174x.  154xC boards come ready to support hard drives

larger that 1 Gigabyte.

Q.  Why can't I use my new CD-ROM/Tape/WORM drive with my

Adaptec HA?

A.  Most Adaptec SCSI HAs are designed to support two HDs and

on some models 2 floppy drives with just a bare board.  Additional

devices require the use of a device driver.

For more information on what your particular HA/kit should support,

contact your dealer/distributor or look at KITS.TXT on this BBS

for an overview.  Spec sheets are available on Adaptec's Interactive

FAX system.

Q.  Why when I boot do I see a message that says

something like "SCSI BIOS not installed", or I see a message that

says "Drive C already installed" or "Searching for Target 0"

and my computer hangs here for about 60 seconds before going

on?

A.  The BIOS on Adaptec HAs is used primarily for booting off

of attached SCSI HDs set to ID 0 (Target 0).  SCSI ID of the HD

is set with jumpers or switches located on the HD.

If the HA you are using is not being used to control a bootable

SCSI HD, and the HA has been working with the attached devices

and their associated drivers, then disable the BIOS on the HA

you are using.  Consult your HA User's Guide, or Installation

Pamphlet for the jumper that enables the BIOS, or the software

switch that needs to be set to disable the BIOS.  152x and 154x

boards can have the BIOS disabled with jumpers, 1640 and 174x

boards BIOS' enabling is controlled by software.  The 154xC

board's BIOS address is controlled by DIP switched located at the

top edge of the board.

Q.  Why does my 174x board always come up in the configuration

as being in Standard Mode, and what is the difference between

Standard Mode and Enhanced Mode?

A.  The default of the configuration files and overlay files used

to configure the 174x is the most compatible mode, Standard.

Standard Mode is equivalent to 154x emulation.  Standard Mode

requires IRQ, DMA and I/O port definitions and policing by the

installer.  Only 4 174x's can be used if all are in Standard mode

because of the number of available DMA channels for use in this mode.

Enhanced Mode allows the installer the luxury of not having to

worry about the DMA, IRQ and I/O address, as this is all taken

care of by the configuration and EISA motherboard.  Enhanced mode

also is the only mode that supports Fast-SCSI (up to 10 Meg/sec.

and 32 bit addressing).

Up to 4 Gig of RAM is supported, with up to 12 HAs being allowed,

with the limit being imposed by the number of EISA slots.

Standard BIOS supports 2 hard drives with support for more hard

drives or other device types coming from device drivers.

Standard BIOS requires the use of ASPI4DOS.SYS for DOS ASPI management.

Enhanced BIOS supports up to 7 hard drives of up to 8 Gigabyte

each, with support for other devices coming from device drivers.

Enhanced BIOS requires the use of ASPIEDOS.SYS for DOS ASPI management.

Support for drives grater than one Gigabyte can only be achieved with

174x BIOS part number 450216-00A or greater with the ASW-C174

version 3.1 configuration files, available on this BBS in the

file called ASWC174.EXE.  As with any Adaptec host adapter,

hard drives controlled by the BIOS need to be partitioned with

DOS's FDISK for use with DOS.

Q.  Why can't I configure my 174x in Enhanced Mode, seeing

the EISA motherboard error "ID mismatch in slot X", or why

do I always see the error "Host Adapter Initialization Failed!"

when I configure the card for Enhanced Mode?

A.  As the EISA standard evolved,  the

differences between different manufacturers propagated.  The

174x family of boards are well designed, high performance boards

that need some special attention to get them installed in a

few EISA motherboards.  Some of the Adaptec Technical Support

situations that have occurred include:

1) When using the AMI ECU program, the installer should

verify that they are using version 1.4C or higher of

that ECU program.  When using this ECU program, the

installer should verify that they are using the

correct CFG file needed for their type of 174x

board, and that the file ADP0000.OVR (dated 6/4/92)

exists in the same directory that the ECU

program and CFG files are in.  When using the AMI ECU

program earlier than 1.4C, the installer will not be

able to configure the board in Enhanced Mode, using

the ASW-C174 utilities v3.0 or later.

To configure the 174x, put the appropriate !ADPxxxx.CFG

file from the ASW-C174 configuration set into the same

directory that the CFG.EXE program is run from.

The files are:

!ADP0000.CFG            AHA-1740 (no letter)

!ADP0001.CFG            AHA-1740A

or AHA-1742A w/floppy

disabled

!ADP0002.CFG            AHA-1742A

!ADP0400.CFG            AHA-1744

Also, in the same directory, be sure that the

ADP0000.OVR file exists.

2) When using some Compaq DeskPro, SystemPro, or similar

computers that have the CPU mounted on a special board, the

installer should take care to minimize the number

of boards between the CPU board and the Adaptec

174x board.  It seems that when other EISA boards

exist between the Adaptec HA and the CPU board, the

Enhanced Mode configuration will not complete, or

the board will report an error indicating that the

HA could not be initialized.  This seems to be a

situation that is not consistent, and is usually only

associated with EISA communication boards in the

slots between the HA and CPU boards.

3) It seems that some 50Mhz and faster motherboards have

increased the response time of the INT signal from

the adapter cards.  If your 50MHz or faster

EISA motherboard gives you the error "Failed to

Initialize Host Adapter" after configuring it for

Enhanced Mode, then follow these steps.

Using a DOS editor, open the CFG file (it's ASCII text)

being used.  Somewhere around line 120 you will see a line that

reads "CHOICE = ENHANCED MODE".  Under this a few lines

will be the line "TRIGGER = EDGE" or "TRIGGER = LEVEL".

If this line says EDGE, change EDGE to LEVEL, this is the

standard setting for MOST EISA motherboards.  If it

says LEVEL, then try using EDGE.

WARNING!!!  This fix has only been used in less than

one percent of the cases, so don't use this as a catch all

fix.  The recommended and default setting is LEVEL.

###################################################################################################################################################################################################################################################################################################################################################################################################################################################################